Security Assessment Findings

em-trak FA170 Class A AIS Transponder

Serial/NMEA 0183 interface: vendor disclosure

# 1. Overview

This document reports security findings against the em-trak FA170 Class A AIS transponder, tested through its serial (NMEA 0183) presentation interface, the port through which the unit receives GPS and sensor data. It is organized in two parts: one confirmed attack and a set of specification non-conformances that are real violations of IEC 61162-1:2024 and could enable attacks, but whose impact depends on other equipment (chart plotters, recorders, gateways, monitors) that we cannot test and are configuration-specific to each ship.

Reference standards:  IEC 61162-1:2024 (serial data interface / NMEA 0183); IEC 61993-2 (Class A shipborne AIS equipment); ITU-R M.1371 (AIS technical characteristics); SOLAS V/19 (carriage and continuous operation).

Device under test:  em-trak FA170, bench test identity MMSI 311001178.

# 2. Test method

Testing was performed on the bench inside an RF-sealed enclosure using reserved test identities. A valid position (the baseline) was fed to the unit continuously; each test then injected one malformed sentence, or, for acceptance tests, repeated it briefly to mimic an attacker who has taken over the data line. Effects were observed on two independent channels: the unit's own serial output and its VHF radio transmissions captured by a software-defined radio, so an effect seen on VHF is confirmed to leave the unit over the air.

# 3. Confirmed attack: denial of position (oversized-sentence DoS)

Severity:  High

Input sent:  A valid position sentence padded with extra characters to lengths from 90 up to 16,384 characters (the standard caps a sentence at 82). Example: $GPRMC,120000.00,A,4330.0000,N,07130.0000,W,0.0,90.0,180626,,,A,9999...9999*hh

Result observed:  The unit keeps transmitting on schedule (about once a second on serial, and at its normal slot on VHF), but while it ingests the oversized sentence, it is starved of valid GPS input, so its position reports carry the “no position available” sentinel (latitude 91 / longitude 181) instead of a real fix. This no-position interval grows with input length: negligible below about 2,000 characters, rising to about 61 seconds at 16,384 characters, and was seen on both the serial output and the VHF transmissions. The interval tracks the time taken to write the oversized input, that is, how long the real GPS feed is held off the shared line.

Why it is an attack:  Denial of position. One oversized sentence monopolizes the shared serial line for as long as the attacker chooses to make it, starving the transponder of GPS so that it broadcasts “no position” for that interval; repeating it keeps the vessel un-plottable. This is not radio silence: the unit’s MMSI is still heard, but with no position, the vessel cannot be shown at a location on other ships’ or VTS collision-avoidance displays. A longer input produces a longer outage.

Why it is self-contained:  This attack needs nothing beyond the ability to put data on the transponder’s input line. It does not depend on any other equipment, any monitor, or any assumption about how downstream devices behave.

Specification violated:  IEC 61162-1:2024, 7.3.1: “The maximum number of characters in a sentence shall be 82.” The resulting loss of continuous valid-position reporting also defeats the Class A carriage requirement (IEC 61993-2 / SOLAS V/19).

# 4. Additional non-conformance (potential impact, not confirmed)

The items below are confirmed violations of IEC 61162-1:2024. Each could plausibly enable an attack, but confirming that would require testing equipment downstream of the transponder, which we did not have. They are reported so the vendor can close the gaps.

| Non-conformance | What we observed | IEC clause | Possible impact (not confirmed) |
| --- | --- | --- | --- |
| Hidden-sentence injection (bad terminator) | A doubled or swapped CR/LF made em-trak parse and broadcast a hidden second position sentence (seen 5 to 7 times per test). | 7.3.1, 7.4 | Injects a position report that a monitor inspecting only the first sentence would miss. |
| Control byte in the unit's own output | While rejecting some malformed input, em-trak emitted its own $AINAK sentences containing a raw DEL byte (0x7F) with a valid checksum. | 7.1.3 | A downstream device that cannot handle the byte could crash, or stall/drop its feed (denial of service). |
| Malformed characters used as a position | Sentences with a reserved or invalid byte ($, !, ^, ~, DEL) in a field were used as a real fix instead of being rejected. | 7.1.2, 7.1.3, 7.4 | Input validation cannot be relied on; on its own, no more than a plain spoof (which already needs line access). |
| Invalid talker label accepted | A sentence labelled $G1RMC (a digit in the two-letter talker field) was accepted. | 7.3.1, Table 4 | Input-validation gap; possible evasion of a filter keyed on valid labels. |

Per-input detail (reserved and invalid characters injected in a data field). For each byte: whether em-trak used the malformed sentence as a real position, whether that went out over VHF, and whether the unit’s own output stayed conforming:

| Byte in a field | Used as position (serial) | Sent on VHF | em-trak output | Net effect |
| --- | --- | --- | --- | --- |
| $ (0x24) | Yes | Yes | Well-formed | Used as position (also on air) |
| ! (0x21) | Yes | No | Well-formed | Used as position (serial) |
| ^ (0x5E) | Yes | No | Well-formed | Used as position (serial) |
| ~ (0x7E) | Yes | No | Well-formed | Used as position (serial) |
| DEL (0x7F) | Yes | Yes | Well-formed | Used as position (also on air) |
| NUL (0x00) | No | No | Non-conforming ($AINAK, raw DEL byte) | Rejected, but corrupt output |

“Well-formed” output means a valid AIS report that simply carries the false position; “non-conforming” means em-trak’s own output broke the standard. The corrupt output seen for NUL also appeared while em-trak rejected a non-hexadecimal checksum and a lower-case talker label, so it is a general reaction to malformed input. Serial acceptance is measured over many reports per test; VHF has a slower slot, so fewer land in a test window (hence “serial” where the air was not caught).

Of these, the two most likely to matter in practice are hidden-sentence injection and the control byte in the unit’s own output: both put unexpected data onto equipment beyond the transponder. Whether that data causes a real fault depends on how each downstream device parses NMEA, which is the next thing to test.

# 5. Attack surface

All findings target the serial (NMEA) presentation interface. Aboard ship, this bus is shared with GPS, ECDIS, sensors, and multiplexers, so reaching it does not necessarily require physical access: a remote attacker who has pivoted onto a networked shipboard device (for example, an ECDIS or a VSAT-connected system bridged to the NMEA bus) can inject the same sentences. The findings above, therefore, describe remotely reachable behavior, not only local access.

Prepared for coordinated disclosure. Findings for the remaining two vendors will be appended in this same format. Contact: ______________________.